DNA-damaging effects of exposure to radiofrequency electromagneticfields (UMTS, 1950 MHz) in human fibroblasts in vitro

Elisabeth Kratochvil, Claudia Schwarz, Alexander Pilger, Franz Adlkofer and Hugo Rüdiger

Division of Occupational Medicine, Medical University of Vienna, Austria

Contact: elisabeth.Kratochvil@meduniwien.ac.at

# Aim of the work

Our previous research demonstrated that radiofrequency electromagnetic fields caused by GSM signals have genotoxic properties in vitro; there are no corresponding data concerning the recently introduced European mobile communication standard UMTS (Universal Mobile Telecommunication Standard) as yet. A putative genotoxic potential of UMTS signals was investigated in cultured human cells using the alkaline comet assay and the cytokinesis blocked micronucleus assay.

# Materials and Methods

Cultured human fibroblasts (diploid fibroblasts obtained from a male 6 year old donor) were exposed to UMTS test signals (carrier frequency 1950 MHz) in a temperature controlled exposure setup specially designed for blind in vitro exposures of cell monolayers and provided by the IT´IS Foundation (zürich, Switzerland).

Dose dependency:

Continuous exposure for 24 hours was performed in all experiments with SAR values between 0.05 and 2.0 W/kg. Each experiment was conducted in quadruplicate. Using the alkaline comet assay (evaluated by visual classification) 3 slides (500 cells) each per experiment were evaluated.

Time dependency:

Continuous exposure at a SAR value of 0.1 W/kg was performed in all experiments with exposure durations between 4 and 48 hours. Each experiment was conducted in triplicate.Using the alkaline comet assay (evaluated by visual classification) 3 slides (500 cells each) per experiment were evaluated.

Intermittency:

Different intermittency schemes were tested using exposure conditions proved to be optimal in the previous tests (16 hours and 0.1 W/kg). Each experiment was conducted in triplicate. Using the alkaline comet assay (evaluated by visual classification) 3 slides (500 cells each) per experiment were evaluated.

Positive and negative controlls were included in each set of experiments to check for effective operation of the method.

Three different cultured fibroblast cell-lines were exposed, using exposure conditions proved to be optimal in previous test (SAR 0.1 W/kg, exposure time 16 hours, intermittency 5 min on, 10 min off).

Each experiment was tested in triplicate. Using the cytocytokinesis blocked micronucleus test 3 slides (500 binucleated cells each) per experiment were evaluated.

Positive and negative controlls were included in each set of experiment to check for effective operation of the method.

# Results

Dose dependency - exposure to UMTS signals results in a significant increase in DNA strand breaks in vitro - below (SAR 0.1 W/kg) and at the current European safety limit (SAR 2 W/kg).

Time dependency - exposure to UMTS signals at a SAR value of 0.1 W/kg results in a time-dependent increase in DNA strand breaks.

Fig.1 : Continuous exposure of cultured human fibroblasts to UMTS test signals for 24 hours results  in a dose-dependent increase in DNA strand breaks.